# Chakli Machine

# Hardware

## Stepper Motor

## Stepper Driver

### TB6600

#### Features

- Basic stepper driver

### TMC2209

#### Features

- Silent mode

- Sensor-less homing

## Emergency Stop

## Display

## Search alternative to VFD

## Start button

# Firmware

## G-Codes

| G-Codes | Description |
| --- | --- |
| G1001 | Feeder Control |
| G1002 & G1003 | Spiral / Cricle (CW & CCW) |
| G1004 & G1005 | Spiral / Cricle (CW & CCW) with table planning |
| G1006 | Line |

## G1001 - Feeder control

### Parameters

- D - Direction

### Command Requirements

- D - Direction control

- 0 value to stop

- Positive value for forward

- Negative value for reverse

- Stop first when direction set

## G1002 & G1003 - Spiral / Circle (Clockwise & counter-clockwise)

### Parameters

- X - horizontal center location in mm (int)

- Y - vertical center location in mm (int)

- R - inner start radius of circle in mm (int)

- P - number of turns in step of 0.25 (float)

- S - spacing between two parallel arcs in mm (int)

- F - feed-rate at which table is moved (int)

- D - start delay in milliseconds (int)

- C - end delay in milliseconds (int)

- H - hole diameter in mm

### Command requirements

  - Draw spiral staring from X, Y as center location

  - Go to X, Y only if all spiral min-max (top, bottom, left & right) inside table size, or send error as reply.

  - R shall be from 1mm to 50mm, default to 5

  - P shall be from 1.0 to 15.0 with step size 0.25 (90degree), default to 4.0

    - Draw arcs in 90 degrees of incremental

  - S shall be from 1mm to 30mm, default to 9

  - F shall be from 10mm/sec to 100mm/sec (600 - 6000mm/min), default to 25 (1500)

    - Mm/min = 60 * mm/sec

    - Can we set higher feed rate at end to cut?

  - D shall be from 0.0sec to 2.0sec (0 - 2000ms), default to 0

  - C shall be from 0.0sec to 1.0sec (0 - 1000ms), default to 700

  - H shall be from 1mm to 60mm, default to 5

### Drawing requirements

- Check all params are received, if no then send error

- Check all params are in limit, if no then send error

- Calculate & check if spiral fits on table min-max, if no then send error

  - Formula

    - Step Size = S + H

    - Spiral Size = R + (round(P) * Step Size)

- For even half arcs (upper) center is at X, Y

- For odd half arcs (lower) center is at X-(R*0.5), Y

## G1006- Line

## G1007- Stepper Configuration

Pitch

## G1008- Table Configuration

Size

## TMC 2209 Sensor-less homing (StallGuard2)

## Start Button

# Mobile / PC Software

## Wizard for setup

Optimize app flow for better usability

# Issues

## Hardware

## Firmware

- Feeder G-code not available

- Spiral G-Code does not match requirements

## Mobile or PC App

- Update mobile app to support only above issues